PART 1 - GENERAL

1.1	Scope of Work

.1	Work of this section shall be performed in accordance with the

requirements of the Contract Documents including but not limited to Instructions to Bidders, Agreement and General Conditions and General Requirements.

.2	Providing new custom, aluminum stile and rail glass doors and

removable center mullion, fit into existing aluminum frame.

1.2	Submittals

.1	Submit test reports, shop drawings and color chips for review.

.2	For closeout:   O & M data, touch up paint.

1.3	Related Work Specified Elsewhere:

.1	Section 08810 – Glazing.

1.4	Measurements

.1	Contractor shall field measure all critical existing frame openings

and adjust door sizes to fit.

1.5	Quality Assurance

.1	Provide test reports from AAMA accredited laboratories certifying the

performance as specified in 1.5.3.

.2	Test reports shall be accompanied by the door manufacturer’s letter of

certification stating that the tested door meets or exceeds the referenced criteria for the appropriate ANSI/AAMA type:

.3	Condensation Resistance Test (CFR):  Shall not be less than 44.

.4	Final assembled door and framing system shall be engineered for all

deadloads and for a minimum of 20 psf positive or negative wind pressure.  Reinforce members as required.

PART 2 - PRODUCTS

2.1	General

.1	Exterior set of “Stile and Rail” doors and frames to be Custom Kawneer

500 Doors and Framing, or Architect approved equal.

2.2	Materials

.1	Aluminum door, frame and trim shall be extruded of 6063-T5 or 6063-T6

aluminum alloy and temper and shall have a minimum thickness of .125”.

.2	Stile and Rail Door

.1	Exterior door to be 1¾” thick x 5” w door stiles (rails as

necessary).  Provide stops and EPDM gaskets for ¼” thick glazing.

.2	Provide bottom rail weatherstripping on the inside of each door and

flush aluminum threshold 6”w.

.3	All screws and miscellaneous fasteners shall be aluminum or non-

magnetic stainless steel.

.4	Provide all miscellaneous accessories, custom break metal, etc. required for a complete finished system.

2.3	Fabrication

.1	Except to the extent that more specific or stringent requirements are

indicated, provide manufacturer’s standard fabrication that complies with indicated standards.

.2	Stile and Rail Doors:

.1	Door stiles and rails shall have hairline joints at corners.

.2	Exterior corner construction is true mortise and tenon for physical

interlock between the rails and stiles.

.3	Interior corner construction shall be joined by heavy concealed

reinforcement brackets with screws and shall be deep penetration and fillet welded.

.3	All doors shall have an adjusting mechanism in the top rail to provide for

minor clearance adjustments.

2.4	Finish

.1	PPG Duranar organic finish conforming to AAMA 2605-02: warrant for

fifteen (15) years against chipping, peeling, cracking or fading.

2.5	Hardware (US26D or as approved by Project Architect)

.1	Hinges (Each Door Leaf):  Rotan – continuous gear hinges, anodized

color to match frame, mortised, heavy duty.

.2	Exit Device (Each pair of Exterior Stile and Rail Doors):  Rim type, all

brass construction.  VonDuprin, one 99NL and one 99EO (cylinder by hardware).

.3	Electric Strike at Two Doors:  Folger Adams 310-4.

.4	Closers (Each Door Leaf):  LCN 4110 Series Parallel Arm with metal

cover, finish color to match door.  (Set for 110º opening).  Mounted on interior side.  Provide mounting plate.

.5	Overhead Stops (1 for each door set at 100º opening).  (Provide wall

stops where wall is available within 120º opening).

.6	Door Sweep:  Reese 964C Nylon Brush Weatherstripping.

.7	Miscellaneous hardware as required by manufacturer.

PART 3 - EXECUTION

3.1	Installation

.1	Install all hardware per manufacturer’s instructions and required

conditions.  Adjust/fit to operate freely, smoothly and properly for its function.  Ensure rigid securement.

3.2	Keying

.1	Cylinder provided by Hardware Division, keyed into building master key

system.  Door to match school key.

3.3	Protection

.1	Cover door, hardware, etc. with heavy cloths until painting is completed.

Prior to completion of the project, examine door and movable parts, adjust as required, and leave hardware in good working order free from defects.

END OF SECTION