## PART 1 – GENERAL

  - RELATED DOCUMENTS

- Drawings and general provisions of Contract, including General and Supplemental and Division 1 Specification sections, apply to the Work of this section.

  - DESCRIPTION OF WORK

- “Hardware” includes items known commercially as builder’s hardware which are required for swing, sliding and folding doors except special types of unique and non-matching hardware specified in the same section as the door and door frame.

2.	All hardware furnished in connection with doors bearing Underwriters’ Labels or where necessary to meet building codes, federal guidelines for non-discrimination on the basis of disabilities (ADA), and special requirements will be in strict accordance with conditions established by the authority having jurisdiction and subject to approval of that authority.

3.	Locking and latching devices on doors from classrooms and all other rooms accessible to students shall be of a type that will always allow free and unobstructed exit from that space by a familiar means such as a knob, lever or exit device.

4.	All building exits and any door which is a required means of egress from an area having an occupant load of 50 persons or more may be provided with a latch or lock device only if it is an approved exit device, U.L. labeled for “ACCIDENT HAZARD”

## 1.03	SCOPE OF WORK

1.	The hardware supplier shall furnish and delivery to the project all items or architectural hardware hereinafter specified in the plans and specifications, or where necessary to assure the proper operation of all doors and wherever hardware is scheduled.  Items not specifically mentioned but necessary to complete the work shall be furnished, matching in quality and finish the items described or specified. Supplier is responsible for proper coordination of all finish hardware with related sections to insure compatibility of products.

2.	Items included in this section are as follows:

All Door Hardware

Thresholds

Weatherstripping

Key Cabinet

1.04	RELATED WORK IN OTHER SECTIONS

1.	The following items are specified in other Divisions of the Specifications:

Cabinet Hardware

Steel/Expansion Joint Covers/Sill Plates

Window Hardware

Toilet Partition Hardware

Identifying Devices

Marble Thresholds

1.05 	QUALITY ASSURANCE

1.   	Obtain each kind of hardware (latch and locksets, hinges, etc.) from only one manufacturer, although several may be indicated as offering products complying with requirements.

2.   	Supplier:  A recognized builders hardware supplier who has been furnishing hardware in the project vicinity for a period of not less than 2 years, and shall have in his employ or at 	his disposal a qualified Architectural Hardware Consultant certified by an independent organization such as the Door and Hardware Institute or the Society of Architectural Hardware Consultants.  The hardware consultant shall be available to meet with the architect and/or General Contractor upon reasonable notice to: 1) review the hardware schedule submittal, 2) coordinate with other trades, 3) make final adjustments to closers and exit devices and 4) assist electrician with final testing of electric locking devices.

1.06 	SUBMITTALS

1.   	Submit manufacturers technical information for each item of hardware. Include whatever 	information may be necessary to show compliance with requirements, and include instructions for installation and for maintenance of operating parts and finish.  Transmit copy of applicable data to Installer.

2.   	The hardware schedule shall consist of the actual product series numbers.  Bidders are required to follow manufacturer's catalog requirements for actual size of door closers, brackets and holders.  All door sizes are to be noted on the hardware schedule and all hardware shall be in strict accordance with height, width, and thickness requirements.  Where closers or overhead door stops are scheduled, the hardware schedule shall indicate 	the degree of opening for each door affected.

3.   	The hardware supplier shall submit to the architect for approval six (6) copies of the complete hardware schedule within fifteen (15) days after award of the contract.  No hardware shall be delivered until the hardware schedule has been approved by the 	architect.

4.   	The schedule shall follow requirements of the specification and shall indicate type, manufacturer's name, catalog number, location and finish of each item to be furnished, all in accordance with the Door & Hardware Institute "Architectural Hardware Scheduling Sequence and Format".  "Vertical" scheduling format only. "Horizontal" schedules will be returned "Not Approved." Preface sheet listing category only and manufacturer's names of items being furnished as follows:

CATEGORY	SPECIFIED		SCHEDULED

Hinges		Manufacturer A	Manufacturer B

Locksets	Manufacturer X	Manufacturer X

Door Closers	Manufacturer Q	Manufacturer Z

Exit Devices	Manufacturer C	Manufacturer F

5.   	Hardware for aluminum doors shall be scheduled in a separate section of the schedule.

1.07	SUBSTITUTIONS AND SAMPLES

1.	Preferred Products:  Due to successful performance, durability and maintainability, the Plant Maintenance Department of the Rochester City School District stocks parts, is trained to repair, and uses the following preferred products.

2.	Equivalents:  In regards to the “OR EQUAL CLAUSE” of the General Conditions of this contract, equivalents of equal or better quality may be made if approved by the District’s Design Group.  If the Bidder intends to use a equivalent on this project, the equivalent must be submitted to the District’s Design Group for approval prior to the award of the Contract.  Such request for equivalents by the Bidder shall be accompanies by physical samples of those items he proposes to substitute. Bidder shall include letter indicating which specific product feature(s) called for in the specification which the proposed substitution does not provide. Thereafter, if no requests for equivalents are presented prior to award of the Contract, the District must assume that the Bidder has intended in his bid to use these specified products.

3.	Sample Request:  Any request by the architect for samples shall be furnished by the hardware supplier no later than ten (10) days after said request is received.

## 1.08	MARKING, PACKING, DELIVERY AND STORAGE

1.	All locks, exit devices, door closers, overhead door holders, hinges, kickplates, pulls and push plates, threshold and other similar items shall be individually packed in separate, suitable original containers as furnished by the hardware manufacturers.  Each container shall be clearly marked with items numbers, article numbers and names, corresponding to that listed in the hardware schedule.

2.	Small miscellaneous items, such as door stops, coat and hat hooks and door silencers that would not require specific location identifications, may be quantity packed if properly labeled with item numbers, etc.

3.	The Contractor shall check the hardware upon delivery with the aid of a representative of the hardware supplier’s firm.  The Contractor shall be responsible for the proper storage of all hardware until ready for application.

1.09	JOB CONDITIONS

1.   	Coordinate hardware with other work.  Tag each item or package separately, with identification related to the final hardware schedule, and include basic installation instructions in the package.  Furnish hardware items of proper design for use on doors and frames of the thicknesses, profile, swing, security and similar requirements indicated, as necessary for proper installation and function.  Deliver individually packaged hardware items at the proper times to the proper locations (shop or project site) for installation.

2.   	Furnish hardware templates to each fabricator of doors, frames and other work to be factory-prepared for the installation of hardware. Check the shop drawings of such other work, to confirm that adequate provisions are made for the proper installation of hardware. All template information shall be in accordance with the DHI handbook, “Recommended Procedure for Processing Hardware Schedules and Templates”.

## PART 2 - PRODUCTS

2.01 	MATERIALS AND FABRICATION

1.   	The drawings show the direction of slide, swing or hand of each door leaf.  Furnish each item of hardware for proper installation and operation of the door movement as shown.

2.   	Produce hardware units of the basic metal and forming method indicated, using the manufacturer's standard metal alloy, composition, temper and hardness, but in no case of lesser (commercially recognized) quality than specified for the applicable hardware units by FS-H-l06, FS FF-G-lll, FS FF-H-ll6 and FS FF-H-l2l.  Do not furnish "optional" materials or forming methods for those indicated, except as otherwise specified.

3.   	Manufacture hardware to conform to published templates, generally prepared for machine screw installation.  Do not provide hardware which has been prepared for self-tapping 	sheet metal screws, except as specifically indicated.

4.   	Furnish screws for installation, with each hardware item.  Provide Phillips flat-head screws except as otherwise indicated.  Finish exposed (exposed under any condition) screws to match the hardware finish or, if exposed in surfaces of other work, to match the finish of such other work as closely as possible, including "prepared for paint" in surfaces 	to receive painted finish.

- Provide concealed fasteners for hardware units, which are exposed when the door is closed, except to the extent no standard units of the type specified are available with concealed fasteners.  Do not use through bolts for installation where the bolt head or the nut on the opposite face is exposed in other work, except where it is not feasible to adequately reinforce the work.

6.   	Furnish a complete set of specialized tools as needed for Owner's continued adjustment, maintenance, and removal and replacement of builder’s hardware.

7.	The Contractor shall check the hardware upon delivery with the aid of a representative of the hardware supplier’s firm.  The Contractor shall be responsible for the proper storage of all hardware until ready for application.

## 2.02	MANUFACTURERS

1.	Numbers listed are from the catalogs of the following manufacturers:

Schlage Lock Company	San Francisco, CA

The Stanley Works	New Britain, CT

Select Products	Kalamazoo, MI

LCN	Princeton, IL

Von Duprin, Inc.	Indianapolis, IN

Glynn Johnson Corp	Indianapolis, IN

Rockwood Mfg.	Rockwood, PA

HB Ives	New Haven, CT

Builders Brass Works	Los Angeles, CA

National Guard Products	Memphis, TN

Wooster Products, Inc.	Wooster, Ohio

## 2.03	FINISH

1.	Finish of hardware shall be:

.1	Hinges US____ @ all interior wood doors, USP @ all interior hollow metal doors, ________ @ all exterior wood & hollow metal doors.

.2	Closers ________ powder coat, equal to LCN ____________ finish.

.3	Trim (push, pull, kickplates)  US_______ except on room side of doors to toilet and locker rooms.  Room side of toilet and locker rooms shall be US32D.

.4	Exit Devices

US_______ lock and end stile cases

US_______ trim

__________ anodized finish on devices mounted on aluminum doors

.5	Weather and sound seals; Dark Bronze anodized or lacquer

.6	Thresholds; Abrasive cast Aluminum

.7	Silencers; Grey

## 2.04	KEYING		NEW KEYING SECTION

1.	Keys and Locking System

.1	All keys, cores and cylinders to be Everest restricted “D” family as manufactured by Schlage Lock Company (owners existing system).

.2	All keys shall conform to Patent #5, 715, 717 (Schlage Everest).

.3	The keying system used must have a patent expiration date of no less than 7/1/2014.

.4	The keying system used must have no less than 10 keyway sections to be familied together under one great grand master.

.5	All cylinders must be keyed to the great grand master system following the specific instructions of the school district or its appointed liason.

.6	All codes must be generated at the lock manufacturer mastery department and copyrighted.

.7	All keying shall be done at the factory or by a qualified locksmith with factory bitting under the direction of a certified Architectural Hardware Consultant (AHC).

.8	All locks and lock cylinders must accept large format interchangeable core as well as standard core cylinders where applicable.

.9	Provide interchangeable core cylinders at all exterior doors, standard core cylinders on all interior doors.

.10	Provide 3 change keys per lock, 5 master keys for each master system and 5 grandmasters for each grandmaster system.  Furnish one extra key blank per lock.

- Keying Schematic and Layout

.1	Lock manufacturer shall successfully conduct keying meetings to ensure proper keying of all locks and cylinders.

.2	The keying schematic shall conform to district keying system requirements in regard to proper lock manufacturer, keyway, code generation and master key order placement.

.3	Hardware supplier shall submit keying schedule for approval prior to placing an order for the locks.  Furnish a bitting list to the school district before installation of permanent cylinders.

3.	Locks and Cylinders

.1	Any lock manufacturer used must make housings for standard core cylinders as well as large format cores.

.2	Lock manufacturer’s representative must assist in keying locks and code generation in such a way that it will conform to all above specifications.

.3	Lock manufacturer’s housing must be able to accept Schlage Everest UL437 listed cylinder.

.4	Mortise locksets shall be used on all new doors and for retrofit of existing doors with old mortise lock cutouts.

.5	Mortise locksets shall be L9000 series as manufactured by Schlage Lock Co. Trim design shall be 93 lever with A style rose unless specified otherwise. 35H series with 3H trim by Best Access Systems is an acceptable equal.

.6	D series cylindrical locksets as manufactured by Schlage Lock Co. shall be used when retrofitting existing doors with cylindrical lock cutouts.

.7	Furnish a key cabinet, Telkee Model AWC 250-S

## 2.05	MATERIALS As listed below, unless indicated otherwise in Hardware Sets

## 1.	Hinges – Stanley, McKinney, Hager

.1	All CB1900 (EXCEPT as noted)

.2	(3) hinges up to and including 90” door height

.3	(4) hinges for doors over 90”

.4	Width of hinge = 4½” except as noted, furnish wide throw hinges where trim conditions require.

.5	Height of hinge = 4½”up to 38” door width - 5” on doors over 38”

.6	ALL hinges on corridor doors shall be NRP (non-removeable pin)

## 2.	Continuous Hinges – Select Products, Roton

.1	Hinge shall be a pinless assembly of three interlocking extrusions applied to the full height of the door and frame without mortising.  The door leaf and jamb leaf shall be geared together for the entire length of the hinge and joined by a channel.  Hinge knuckle shall be monolithic in appearance.  Continuous hinges with visible knuckle separations are not acceptable.  Vertical door loads shall be carried on minimum 3/4" acetal bearings through a full 180 degrees. All (HD) heavy-duty hinges shall have a minimum of (32) acetal bearings supplied on a (84”) tall continuous hinge.

2.	Screw hole locations on door leaf and jamb leaf to be templated.

### .3	Locksets- Schlage, Corbin Russwin ML2000 Series, Sargent 8200 Series, Best 45H

### .1	Locksets shall be Schlage L9000 series mortise type and must have the following features:

.2	Hand of lock can be changed without disassembly of lock case.

.3	Lever support springs are replaceable without disassembly of lock case.

.4	Outside spindles on locking functions on sets with lever handle trim shall be of a type which is separate from the inside spindle. Threaded swivel connection between outside and inside lever trim is unacceptable.

.5	Trim to be applied by threaded bushing. Exposed fasteners shall not be acceptable.

.6	Manufacturer shall provide universal lock chassis, one lock case for knobs or levers.

4.	Lock Trim

.1	Locksets shall be furnished with lever handle trim (lever design to be determined on a per job basis). Levers shall be solid in construction, 4-¾” minimum length. Roses shall be heavy wrought, 2-1/8” maximum diameter.

.2	All locks to be furnished with (6) pin cylinders as specified above. Cylinder cams shall match the lock manufacturer’s template.

5.	Deadlocks – Yale 300 Series – (Accurate, Corbin/Russwin, Sargent)

6.	Exit Devices – Von Duprin, Corbin Russwin ED4000/ED5000 Series, Sargent  80 Series

.1	99 Series except as noted

.2	Furnish RG27 series rod guards at all vertical rod devices (removable mullions are preferred).

.3	Furnish glass bead kits at fire doors wherever the glass area falls below 42” above finish floor.

.4	All devices shall incorporate fluid damper or similar device, which will slow down the touchbar return, thereby eliminating the noise commonly, associated with panic bars.

.5	Strikes at pullman type latches shall be roller type.

.6	The latchbolt shall be molly coated, a self-lubricating coating, which reduces friction and wear.  Latchbolt shall have a deadlocking feature.

.7	Center Case: Shall be interchangeable with all functions.

.8	Mechanism Case: Shall be 1/8" wall thickness, extruded aluminum.

.9	Mechanism End Cap: Shall be cast zinc or forged aluminum and have a minimum thickness of .125". Plastic or metal stampings will not be acceptable.

.10	All internal parts shall be zinc dichromate coated.

.11	Strikes:  Shall be roller type and come complete with a locking plate to prevent movement.

.12	Trim:  Shall be heavy-duty type and fastened by means of concealed welded lugs and thru bolts from the inside. Knob and lever trim shall be forged brass with a minimum average thickness on the escutcheon of (.130). Plate on pull trim shall be minimum average thickness (.090) and have forged pulls and thumbpieces.

.13	Lever trim shall have breakaway feature.

7.	Closers – LCN, Norton 7500 series, Sargent 351 Series, Dorma TS83 Series

.1	Furnish LCN Model #4111 with standard metal cover, EXCEPT as noted.

.2	Indicate degree of opening for mounting closer for each door on hardware schedule.

.3	Furnish with each closer a full size screw location template for correct type of mounting and degree of opening.

.4	Furnish thru-bolts and ceal nuts (NOT GROMMETS) at all wood fire doors and as may be required by carpenter for correct fastening.

.5	Metal cover shall be fastened to closer body with tamper resistant screws. Closer arm shall be fastened to frame with tamper resistant screws.

.6	All closers shall meet or exceed the ANSI 117 standards.

.7	Furnish all adapter plates, brackets, and related closer accessories required by manufacturer for mounting closers on the doors and frames as detailed.

.8	All manual door closers shall be certified to exceed ten million (10,000,000) full load operating cycles by a recognized independent testing laboratory.

.9	All manual closers shall carry a manufacturers ten (10) year warranty.

.10	All closer with electrical or pneumatic components shall carry a manufacturers two (2) year warranty.

.11	Fully hydraulic, rack and pinion action with high strength cast iron cylinders and one piece forged steel pistons.

.12	Hydraulic fluid of a type requiring no seasonal adjustments for temperatures from 120º F (49º C) to -30º F (-35º C).

.13	Pinion shaft minimum diameter of 11/16".

.14	Hydraulic regulation controlled by tamper-proof, non-critical screw valves, adjustable with a hex wrench.  Separate adjustments for backcheck, general  speed, and latch speed. Backcheck shall be properly located for protection of the door, frame, and applied hardware.

.15	Forged steel main arms.  Forged steel main and forearms at PA application. Arms at PA location shall be vandal proof design. Any PA arm that can be taken apart in the field shall not be acceptable.

8.	Push/Pull Plates – Rockwood Mfg. (Ives, Baldwin) Except As Indicated In Hardware Types Furnish:

.1	Push Plate #70 - 3-½” x 15”

.2	Pull Plate #110 x W

9.	Kickplates – Rockwood Mfg. (Ives, Baldwin)

.1	(.051”) thick.

.2	8” high except as noted.

.3	Width of door less 2” on push side.

.4	Width of door less ½” on pull side.

.5	Width of door less 6” on doors with surface vertical rods.

10.	Stops – Glynn Johnson( Ives)

.1	FB13 floor stops at all doors EXCEPT: 60W where knob contacts wall or Glynn-Johnson (Rixson) 450 series OH Stop where floor and wall stops are impractical.

11.	Overhead Door Holders – Glynn Johnson (450 Series),Rixson

12.	Thresholds – Wooster, Pemko, Zero, Reese

.1	Furnish #115-4” Alumogrit - notched for stops except as noted or detailed otherwise.

.2	Furnish stainless steel screws and lead anchors for fastening.

13.	Sweepstrips – National Guard (Reese 964, Seal-Eze)

.1	Nylon Brush type #C607 x width of door

.2	Polypropylene UNACCEPTABLE

14.	Weatherstripping – National Guard, (Reese 961, Seal-Eze)

.1	Nylon brush type A625 – head & jamb

.2	Polypropylene UNACCEPTABLE

15.	Soundproof Gasketing – National Guard (Reese)

.1	Wherever soundproofing is shown or listed in the specifications or drawings, furnish:

.1	103N adjustable gasketing – mitered at head and jambs

.2	420N automatic door bottom

16.	Manual Flush  Bolts – Glynn Johnson (Door Controls, H.B. IVES)  FB6

17.	Automatic Flush  Bolts – Glynn Johnson (Door Controls, H.B. Ives)

.1	FB30 (metal doors) FB40 (wood doors)

.2	Furnish DP2 dustproof strike at bottom bolt

18.	Self-Latching Bolts - Glynn Johnson (Door Controls, H.B. IVES)

FB50 (metal doors), FB60 (wood doors)

.1	Furnish DP2 dustproof strike at bottom bolt

19.	Coordinators - Glynn Johnson (Door Controls, H.B. Ives)

.1	Furnish C O R Series with required brackets and CB-1 carry bar.

20.	Electromagnetic Stops & Holder – LCN SEM Series, Rixson, Dorma

Interior Non-Locking – Not Rated Single Doors

1	Set	Hinges				CB1900 4.5”

1	Set	Passage set				L9010

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Single Toilet - Not Rated

1	Set	Hinges				CB1900 4.5”

1	Set	Privacy Set				L9040

1	Only	Stop				60W/FB13

1	Set	Privacy Set (Thumbturn Both Sides)	L9040 x XL11-446

Private Office – Not rated

1	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9050

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Store Rooms & Classrooms Off  Corridor – Not Rated

1	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9070

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Store Rooms & Janitors Closets - Not Rated

1	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9080

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Closets – Not Lockable – Not Rated

1	Set	Hinges				CB1900 4.5” X 4”

1	Only	Pull				108

1	Only	O.H. Stop & Holder, Friction Type	450F

3	Only	Silencers				64

Closet – Lockable – Not Rated

1	Set	Hinges				CB1900 4.5” X 4”

1	Only	Deadlock				313ST

1	Only	Pull				108

1	Only	O.H. Stop & Holder, Friction Type	450F

3	Only	Silencers				64

Interior – Not Lockable – Rated

1	Set	Hinges				CB1900 4.5”

1	Set	Passage Set				L9010

1	Set	Closer				4111 METAL

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Faculty Toilet

1	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9060

1	Set	Privacy Bolt				K250V

1	Only	Closer				4111 METAL

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Classroom, Inswing  - Rated

1	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9070

1	Only	Closer				4011T METAL

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Store Rooms, Janitor Closets On Corridor – Rated

1      Set                Hinges				CB1900 4.5”

1      Set                Lockset				L9080

1      Only             Closer				4111 METAL

1      Only             Stop				60W/FB13

3      Only             Silencers			64

Executive Office w/Privacy Door Release

1	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9050

1	Only	Closer/Elec. Release H.O.		4040SE (PULL SIDE) X

TRANSFORMER 110/24

1	Only	Push Button Release (@ Desk)	PB720

1	Only	Stop				60W/FB13

3	Only	Silencers				64

NOTE: WIRING BY ELEC. CONTRACTOR

Classroom, Utility – Outswing – Rated

1	Set	Hinges				CB1900 4.5” NRP

1	Set	Lockset				L9070

1	Only	Closer				4111 METAL

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Storage, Janitor Closet – Rated – Outswing To Corridor

1	Set	Hinges				CB1900 4.5” X 4”

1	Set	Lockset x Less Outside Trim

(Cylinder Only)				L9080 LESS

1	Only	Flush Pull				BBW 166

1	Only	Closer				4111 METAL

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Storage, Janitor Closet – Not Rated – Inswing – w/ Closer

1	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9080

1	Only	Closer				4011 METAL X H.O.

1	Only	Kickplate				.050 X 8”

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Public Toilet – Rated

1	Set	Hinges				CB1901 4.5”

1	Set	Lockset				L9070

1	Only	Closer				4111 METAL

1	Only	Kickplate				.050 X 8”

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Public Toilet – Multi Stall - Not Rated

1	Set	Hinges				CB1901 4.5”

1	Only	Push Plate				70 – 3.5” X 15”

1	Only	Pull Plate				110 X W

1	Only	Lockset				H7804 X 7800

(ACCURATE)

1	Only	Closer				4111 METAL

1	Only	Kickplate				.050 X 8”

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Stair, Corridor Exits – Rooms w/ Occupancy > 50 Persons

1	Set	Hinges				CB1900 4.5”

1	Only	Exit Device				99L(-F)

1	Only	Closer				4111 METAL

1	Only	Kickplate				.050 X 8”

1	Only	Stop				60W/FB13

3	Only	Silencers				64

Service Entrance

1	Set	Hinges				CB1961 4.5” NRP

1	Set	Lockset				L9080

1	Only	Closer				4111 CUSH METAL

1	Only	Threshold				115 – 4” ALUMOGRIT

1	Set	Weatherstrip				A625

1	Set	Sweepstrip   				C607

Storage From Exterior

1	Set	Hinges				CB1961 4.5” NRP

1	Only	Lockset				L9080

1	Only	Closer				4111 METAL

1	Only	Threshold				115 – 4” ALUMOGRIT

1	Set	Weatherstrip				A625

1	Only	Sweepstrip				C607

Exit Only – Exterior

1	Set	Hinges				CB1961 5” NRP

1	Only	Exit Device				99E0

1	Only	Closer				4111 METAL

1	Only	O.H. Stop/Holder			100H

1	Only	Kickplate				.050 X 8”

1	Only	Threshold				115 – 4” ALUMOGRIT

1	Set	Weatherstrip				A625

1	Only	Sweepstrip				C607

Exit / Entry – Exterior

1	Set	Hinges				CB1961 4.5” NRP

1	Only	Exit Device				99NL

1	Only	Closer				4111 METAL

1	Only	O.H. Stop/Holder			100H

1	Only	Threshold				115 – 4” ALUMOGRIT

1	Set	Weatherstrip				A625

1	Only	Sweepstrip				C607

1	Only	Continuous Hinge			SL21HD

1	Set	Push/Pull Lockset			HL6-9060

1	Only	Closer				4011 METAL X DEL

1	Only	Armor Plate				.050 X 16”

1	Only	Mop Plate				.050 X 8”

1	Only	Stop				60W/FB13

Interior -  Lockable – Rated  (Pr.)

2	Set	Hinges				CB1900 4.5”

1	Pair	Manual Flush Bolts			FB6

1	Set	Lockset				L9070

2	Only	Stop				60W/FB13

4	Only	Silencers				64

Closet – Not Lockable (Pr.)

2	Set	Hinges				CB1900 4.5” X 4”

2	Only	Pull				108

2	Only	O.H. Stop/Holder			450F

2	Only	Friction Catch				21

2	Only	Silencers				64

Closet – Lockable – Not Rated (Pr.)

2	Set	Hinges				CB1900 4.5” X 4”

2	Only	Pull				108

2	Only	Manual Flush Bolts			FB6

2	Only	O.H. Stop/Holder			450F

2	Only	Friction Catch				21

1	Only	Deadlock				313ST

2	Only	Silencers				64

Gym Storage – Not Rated (Pr.)

2	Set	Hinges				CB1900 4.5” X 4”

2	Only	Flush Pulls				95C

2	Only	O.H. Stop/Holder			450F

2	Only	Friction Catch				21

1	Only	Deadlock				313ST

2	Only	Silencers				64

2	Set	Hinges				CB1901

1	Pair	Flush Bolts				FB6

1	Only	Deadlock				313ST

- Only	Push Plates				70 – 3.5” X 15”

2	Only	Pulls				TO BE SELECTED

2	Only	Closers				4111 METAL

2	Only	Kickplates				.050 X 8”

2	Only	Stops				60W/FB13

Storage Off Corridor – Rated (Pr.)

2	Set	Hinges				CB1900 4.5”

1	Set	Lockset				L9050

1	Set	Self Latching Flush Bolts		FB50/FB60 X DP2

1	Only	Closer				4110 METAL

1	Only	Stop				60W/FB13

2	Only	Silencers				64

2	Set	Hinges				CB1900 4.5”

2	Only	Manual Flush Bolts			FB6

1	Set	Lockset 				L9060 X OPEN

BACK STRIKE

2	Only	Closers				4111 METAL X H.O.

2	Only	Kickplates				.050 X 8”

2	Only	Silencers				64

Storage On Corridor – Rated (Pr.)

2	Set	Hinges				CB1900 4.5” X 4”

1	Set	Self Latching Flush Bolts		FB50/FB60 X DP2

1	Set	Lockset				L9080

1	Only	Closer (Active Leaf)			4111 METAL

2      Only	 Stops				60W/FB13

2	Only	Silencers				64

Classroom – Rated (Pr.)

2	Set	Hinges				CB1900 4.5”

1	Set	Automatic Flush Bolts			FB30/FB40 X DP2

1	Set	Lockset				L9070

2	Only	Closers				4111 METAL

1	Only	Coordinator				COR-2

2	Only	Kickplates				.050 X 8”

2	Only	Stops				60W/FB13

Meeting Rooms, Classrooms  - Occupancy >50 Persons – Rated Metal Doors (Pr.)

2	Set	Hinges				CB1900 4.5”

2	Only	Exit devices				9927L-F

2	Only	Cylinders				20-022

2	Only	Kickplates				.050 X8”

2	Only	Smoke Astragals			A605

2	Only	Electromagnetic Holders		SEM7830

Meeting Rooms,Classrooms – Occupancy > 50 Persons – Rated Wood Doors (Pr.)

2	Set	Hinges				CB1900 4.5”

2	Only	Exit devices				9975L (-F) X 9927L(-F)

2	Only	Cylinders				20-022/20-001

2	Only	Closers				4111 METAL

2	Only	Kickplates				.050 X 8”

1	Only	Coordinator				COR-2

1	Only	Carry Bar				CB-1

2	Only	Electromagnetic Holder			SEM7830

Stair, Corridor Exits – Rated (Pr.)

2	Set	Hinges				CB1901 4.5”

2	Only	Exit Devices	 			99L(-F)

2	Only	Cylinders				20-022

2	Only	Closers				4111 METAL

2	Only	Kickplates				.050 X 8”

2	Only	Electromagnetic Holders		SEM7830

1	Only	Removable Mullion			(KR)9954 X 154

(Omit Removable Mullion Where Fixed H.M. Mullion Is To Be Furnished By Others)

Storeroom – Exterior (Pr.)

2	Set	Hinges				CB1961 4.5” NRP

2	Only	Surface Bolts				1712B/1712T

1	Only	Lockset				L9080

1	Only	Closer (Active)				4111 METAL

1	Only	O.H. Stop/Holder (Inactive)		80H

1	Only	Threshold				115-4” ALUMOGRIT

1	Set	Weatherstrip				A625

2	Only	Sweepstrips				C607

2	Set	Center Hung Closer			8200 SERIES H.O

2	Set	Push/Pull Bar				TO BE SELECTED

Vestibule – Non Locking – Aluminum Doors (Pr.)

2	Only	Continuous Hinges			SL24HD

2	Only	Push Bars				TO BE SELECTED

2	Only	Pulls				TO BE SELECTED

2	Only	Closers				4111 CUSH METAL

- Only	Threshold				425BR

Exits Lockable – Aluminum Doors (Pr.)

2	Only	Continuous Hinges			SL24HD

2	Only	Exit devices				9927TL X 9927DT

1	Only	Cylinder				20-001

2	Only	Closers				4111 METAL

2	Only	O.H. Stop/Holder			100H

1	Only	Threshold				425BR

2	Only	Astragals				A605

1	Set	Weatherstrip				A625

2	Only	Sweepstrips				C607

Exit / Entry From Exterior w/ Mullion  (Pr.)

2	Only	Continuous Hinges			SL24HD

1	Only	Exit Device (Active)			99NL

1	Only	Exit Device (Inactive)			99DT

2	Only	Pulls				TO BE SELECTED

1	Only	Cylinder				20-022

1	Only	Removable Mullion w/ Stabilizers	(KR)4954 X 154

2	Only	Closers				4111 CUSH METAL

1	Only	Threshold				425BR

1	Set	Weatherstrip				A625

2	Only	Sweepstrips				C607

Wardrobe Doors

1	Only	Continuous Hinge			780-155HD

1	Only	Overhead  Stop – Friction Type	450F

1	Only	Rod Pull				108

1	Only	Friction Catch				21

END OF SECTION