PART 1 - GENERAL

1.1	Work of this section includes:

.1	Provide playground equipment as indicated on the drawings,

including but not limited to the following:

.1	Play Structures and Apparatus

.2	Rubberized Safety Surface

1.2	Quality Assurance

.1	Manufacturer’s Qualifications:  Provide playground equipment and manufactured by a firm with a minimum of 10 years experience in manufacturing equipment that is similar to that indicated and that has a record of successful performance.

.2	The contractor shall have a minimum of five (5) years experience

related to the work indicated by these specifications.  Submit a list of a minimum of five (5) projects attached to the bid, along with project amounts, names and telephone numbers of Owner's Representative.  Contractor is to be a certified installer of product specified.

.3	All products, equipment and installation shall meet the standards of

U.S. Consumer Products Safety Commission, Handbook for Public Playground Safety and ASTM 1487-07a-e1 Standards.  All manufacturer's product data shall indicate compliance with these requirements.

.4	All equipment shall be manufactured from one source, including all

accessories, fittings and fastenings.

.5	Design Concept:  The drawings indicate the size, profiles and dimensional requirements of the playground equipment required and are based on the specific type indicated.  Equipment by other manufacturers may be considered provided deviations in dimensions and profiles are minor and are submitted for approval to the Project Architect a minimum of 5 days prior to bid date, and do not change the design concept as judged by the Project Architect.  The burden of proof of equality is on the proposer.

1.3	Submittals

.1	Submit manufacturer's product data and installation for approval by

the Project Architect prior to fabrication.  Submit shop drawings and product data showing exact types of equipment and configurations as indicated on drawings and specified.  Colors shall be selected by the Project Architect.

1.4	Product Delivery, Storage and Handling

.1	Materials shall be packed, loaded, shipped, unloaded, stored and protected in a manner which will avoid abuse, damage and defacement.

1.5	Warranty

.1	Playground Equipment Warranty

.1	Playground equipment uprights, hardware, and bolting system shall be guaranteed against structural failure/defects under normal use and service, for the lifetime of the structure.  Such warranty shall cover the material replacement of the equipment.

.2	Playground equipment slides and other rotationally molded parts shall be guaranteed against breakage and splitting for 15 years.

.3	Metal decks, pipes, rails, loops and ladders shall be warranted against structural failure for a minimum of 15 years.

PART 2 - PRODUCTS

2.1	Manufacturers

.1	Playground equipment complying with the project requirements shall

be manufactured by GameTime Playground Equipment, P.O. Box 121, Fort Payne, Alabama 35967, or the equivalent as manufactured by Playworld Systems or Landscape Structures as approved by the Project Architect prior to bid.  Also refer to Section 01 23 00 – Alternates, for inclusion of optional manufacturers..

.2	Authorized GameTime Representative:  Marturano Recreation, Linda Culliton, (585) 223-7398 or (800) 922-0070, Fax (585) 223-0707.

2.2	Playground Equipment – General System Specifications

.1	The uprights shall be factory drilled to ensure accurate placement of components and ease of installation.  Field drilling and measuring are not required.  GT Events are direct bolt products not a clamp system.  All uprights shall receive factory installed aluminum post caps and shall be shipped with a factory applied label indicating proper surfacing level.

.2	All decks and components shall connect to support posts by means of a through-bolt connection for strong, durable connections.  Deck/Collar attachments shall not be acceptable.

.3	Components

.1	General - Rotationally molded forms and shapes shall be constructed of U.V. stabilized linear low density polyethylene with optimum color retention.  Metal supports shall be powder coat finish, galvanized tube and fasteners shall be Xtreme Play, Xscape, Power Scape, Primetime etc. (as indicated on the drawings), powder coating and neoprene gaskets.

.2	Play Structure - Provide equipment type and configuration as shown

on drawings; Powerscape by GameTime, Inc. or the approved equivalent.

.3	Rubberized Safety Surface – Refer to Section 32 18 16 – Synthetic                                                                   Rubber Surface for resilient surface requirements.

.4	Concrete – Refer to Section 03 30 00 – Concrete Work.

.5	Model names and numbers indicated on equipment schedule on

drawings referring to a specific manufacturer are included for comparison purposes only. Equivalent products, meeting project requirements shall be judged by comparison with these products for approval by the Project Architect.

.6	List specific equipment and detailed characteristics.

PART 3 - EXECUTION

3.1	Installation

.1	Comply with the manufacturer's written instructions and as indicated

on the drawings and these specifications.  All products and equipment shall be installed to comply with referenced safety standards and all applicable codes and regulations.

.2	Anchor vertical frames in excavation minimum four foot below grade

to a depth and diameter indicated by manufacturer's instructions.  Set posts in concrete.

.3	Install all equipment plumb, level and secure.  Adjust hardware and

moving parts; test for proper operation and lubricate where necessary.

END OF SECTION