# PART 1 - GENERAL

1.1 	RELATED DOCUMENTS

.1   	Drawings and general provisions of Contract, including General Conditions and Division 1 Specification sections, apply to work in this section.

1.2 	DESCRIPTION OF WORK

.1   	Provide gymnasium wall padding as required to match existing color(s), size and layout.

1.3   QUALITY ASSURANCE

.1	Provide materials from a single source to the greatest extent possible.  Installer shall be a firm with not less than 3 years successful experience in work of this section.

1.4 	SUBMITTALS

.1   	Submit manufacturer's technical data for materials, fabrication and installation, including catalog cuts or anchors, fastenings and accessories.

.2	Submit shop drawings for fabrication and erection of padded wall assemblies.  Field verify existing dimensions, conditions, etc. prior to submittal of shop drawings.

.3	Samples – Submit samples for each type of component specified, including colors for cover fabric to match existing.

# PART 2 -  PRODUCTS

2.1 	MATERIALS

1.   	Provide materials as specified to meet the following criteria and accessories as required to fabricate and install assembly as indicated.

.2	Wall panels shall be as indicated on drawings, size requirement must be field verified.  Corner panels shall “wrap” around corner.

.3	Subject to compliance with project requirements and modifications as indicated, wall padding assembly shall be Vinyl-Nylon Covered Panels by Porter Equipment Co., or the approved equivalent; equivalent products manufactured by Nissen Corporation and Aalco Manufacturing Co. shall be acceptable.

.1  Cover Fabric – 14 oz., flame retardant nylon reinforced vinyl,

mildew resistant, and with a tear strength of no less than 400 psi; color and pattern per Project Architect’s selection.

.2	Interliner Fabric shall be one layer of No. 3 Vonar fire retardant interliner by Dupont in manufacturer’s standard gauge.

.3	Filler Layer – 2” urethane filler, 6 lb. density, UL listed with a flame spread of not over 75 in accordance with ASTM E 84.

.4	Backer Board – ½” CDX plywood backing, fireproofed.

.5	Miscellaneous Materials – Provide materials to complete installation as required for intended use.

.1	Mastic – Adhere core and backer board with manufacturer’s

mastic at rates and coverage as recommended by

manufacturer.

.2	Bolts and Fasteners – Provide manufacturer’s standard fabric and panel fastening devices, of size and spacing recommended by manufacturer’s installation instructions.

2.2	FABRICATION

.1   	Furnish factory assembled panels free of tears, blisters and otherwise inferior methods of fabrication.  Edges are to be true and straight and folds of fabric are to be firm with as few creases as possible.  Cover fabric shall be secured to backer board with edges completely concealed in finished application.

# PART 3 - EXECUTION

3.1 	INSTALLATION

.1   	Install materials in accordance with manufacturer’s printed instructions and as specified herein.

.2	Install wall padding in locations indicated with vertical surfaces and edges plumb, top and bottom edges level.  Fit to adjoining work accurately at borders, cut-outs and penetrations.  Wrap edges of field cut panels and borders; wrap cover fabric entire depth of panel and secure to backer board.  Center layout of panels on wall so that end panels are of equal width.  Arrange panels in selected color patterns.

.3	Attach wall padding to blocking with blind nailing as detailed on drawings.  Space fasteners as required to provide firm assembly under conditions of use.

.4	Remove and replace panels which are damaged and are unacceptable to Project Architect.

END OF SECTION