# PART 1 - GENERAL

1.01 	RELATED DOCUMENTS

1.   	Drawings and general provisions of Contract, including General Conditions and Division 1 Specifications sections, apply to work of this section.

## PART 2 - PRODUCTS

(Not applicable)

# PART 3 - EXECUTION

3.01	USE OF ELECTRICITY

1.	Electric service is available in the present building and may be used by the Contractors without charge.  The Contractor shall make all necessary connections and install fuse blocks in a manner approved by the Project Inspector and acceptable to the Board of Fire Underwriters.  The Contractor shall under no circumstances, connect to a light socket for electric power service.

2.	Temporary lighting shall be supplied by each contractor responsible for work within any specific area requiring renovation as part of this project.  When work involves lighting replacement, the prime contractor supplying replacement shall provide temporary lighting at completion of the demolition/ abatement phase.  All electrical work shall be done in accordance with NEC and all other applicable codes and regulations.

3.02	WATER SUPPLY

1.	Water will be furnished by District without charge.  The Contractors shall provide their own hoses from existing hose bib to points of need with non-leaking hoses.

3.03	SANITARY ARRANGEMENTS

- The Custodian's toilet shall be used as a toilet for all workmen employed on this project.  The Contractor will be held responsible for his share in maintaining this room in a clean, sanitary condition.  No other toilet shall be used as such convenience.

3.04	TEMPORARY ENCLOSURES

1.	Temporary closures for all openings to permit installation and/or operation of systems shall be provided by each trade.  Provide protection for adjacent surfaces exposed to higher than normal use/abuse.  Comply with all applicable regulations.

2.	Temporary doors, hinged and latched with duplicate keys for District shall be provided at all applicable interior temporary enclosures and for exterior enclosures at exit.  Existing demoed doors and hardware may be used (if available).

3.	During exterior renovation work, temporary overhead protection shall be provided for any sidewalks or areas immediately beneath the work site or such areas shall be temporarily fenced off and provided with warning signs to prevent entry, by applicable contractor.

4.	Temporary fences around exterior building or site work construction, any construction supplies or equipment, or debris shall be provided and maintained by applicable contractor.

5.	Any temporary scaffolding or temporary construction, left overnight exterior to the building or otherwise accessible to others, shall be fully screened with solid panels up to a minimum 12’ height above grade, to prevent access and/or climbing to upper levels.  If there for an extended period of time, it shall have gates to allow workers access but be locked at night.

6.	Temporary gates shall always be locked unless a worker is in attendance to prevent unauthorized entry.

7.	While the building is occupied, existing exits required for occupied portions shall be maintained active.  Existing fire alarm, detection, exit and emergency light system shall be maintained operable.

3.05	TELEPHONE SERVICE

1.	The Contractor shall provide his own phone service (mobile), and provide the phone number(s) for contact to the Project Inspector and Architect/Engineer.

3.06	ELEVATOR SERVICE

1.	The Contractor (with Project Inspector’s approval) may use existing freight

elevator as available.  Subject to all protection, cleaning limitations and its weight limits.

3.07	TEMPORARY ODOR CONTROL

1.	The Contractor is responsible for the control of his chemical fumes, gases

and other contaminants produced by welding, gasoline or diesel engines, roofing, paving, painting, etc. to ensure they do not enter “occupied” portions of the building or air intakes.  Provide necessary temporary controls.

3.08	TEMPORARY FIRE PROTECTION

1.	The Contractor shall provide adequate number/type of fire extinguishers for

normal operations, or for all specialized fire protection, as necessary.

3.09	STORAGE/STAGING AREAS

1.	Store materials and stage work only in such locations as approved by the Project Inspector.   Move stored materials when, in the opinion of the Project Inspector, job conditions so require.  All storage/staging areas shall be maintained by each contractor.

2.	Large interior areas in the existing building for storage are generally not

available.  Contractors may expect to have to store some equipment/supplies off site (or on site in temporary trailers).

3.	Storage of materials and/or equipment in corridors, stairs, exits, or on roofs is strictly forbidden.

4.	Store all flammable/combustible materials in a locked fire rated enclosure.  Paint shall be stored in a paint locker.

3.10    MAINTENANCE AND REMOVAL OF TEMPORARY FACILITIES

- All temporary work shall be properly maintained, relocated, upgraded as necessary for project progress, and removed when no longer required.  Restore, patch, repair remaining permanent work.

## END OF SECTION